# Smart Warehouse IoT: Product Requirements Document

Draft v2 for a 24-hour hackathon. Items marked [ASSUMPTION] need team confirmation before the TRD.

## 1. Summary

A smart-warehouse system that tells a warehouse owner how many parcels to expect each day (per product, with a likely range), confirms what actually arrived at a physical ESP32 scan station, keeps an authoritative inventory record in the cloud, and estimates when each batch will sell out. The owner sees all of this in a web app.

Only the final scan station is real hardware. The conveyor, weight sorting, and Walmart-style demand data are simulated. The whole cloud side runs on Supabase.

## 2. Problem and users

Problem: A warehouse owner has little visibility into how many parcels are coming, what has really arrived, and whether stock will sell before it expires.

Primary user: the warehouse owner, who uses the owner app.

Secondary audience: hackathon judges watching a live demo.

## 3. Goals and non-goals

### Goals

- G1. Show the owner the expected number of parcels per product for a given day, with a likely range (forecast).

- G2. Confirm real arrivals at an ESP32 station using a trigger and RFID.

- G3. Store product and batch records (type, weight, expiry) in the cloud as the single source of truth.

- G4. Estimate a sell date per batch and update it as new batches arrive.

- G5. Present expected vs. arrived, stock, and sell dates in the owner app.

### Non-goals

- Measuring real weight (weights are simulated product data).

- A physical conveyor or real weight-based sorting.

- Real expiry or food-safety information (all expiry dates are simulated and labeled as such).

- AI directly controlling hardware.

- Multi-warehouse support, user accounts, or production-scale reliability.

## 4. Demo scenario (golden path)

- The owner app shows today's forecast of expected parcels for each product, with a likely range.

- Item data for a product (for example 2 kg, snacks) is registered with the cloud, either live from Fusion or by a replay script that sends the same data.

- A tagged demo box is placed at the ESP32 station.

- The station detects the box (push button or IR sensor) and reads the RFID tag.

- The ESP32 sends the tag to the cloud, which matches it to the registered item and replies with the product type, weight and a simulated expiry.

- The OLED shows them with a 'simulated' marker, and the LEDs and buzzer signal success.

- The app updates: the arrived count rises against the forecast range, the batch appears in stock, and its estimated sell date is recalculated.

- A second box with a different weight repeats the flow so judges see the update.

## 5. Scope

| Priority | Item |
| --- | --- |
| Must | ESP32 scan flow: trigger, RFID read, scan request to the cloud, OLED and LED feedback |
| Must | Supabase database and API (tables plus Edge Functions) holding products, pending parcels, batches, arrival events and forecasts |
| Must | Item registration through a replay script (a live call from Fusion is second priority) |
| Must | Daily forecast per product, with a likely range, shown in the app |
| Must | Owner app: arrivals, stock by batch, simulated expiry, estimated sell date |
| Should | Expected vs. arrived comparison view |
| Should | Expiry-risk flag when the estimated sell date is later than the expiry date |
| Should | Small on-device table of known tags so the demo survives a Wi-Fi drop (results marked OFFLINE) |
| Could | Logistics chat and analysis AI |
| Could | Servo gate as a visual indicator only (it does no real sorting and never blocks the flow) |
| Could | Owner-configurable settings from the app |

## 6. Functional requirements

### ESP32 station

- FR-1. Detect that a parcel has arrived at the station (one digital input: push button or IR sensor).

- FR-2. Read the RFID tag UID.

- FR-3. Send the tag to the cloud and receive the product type, weight and simulated expiry. If the network is down, fall back to a small on-device table of known tags and mark the result OFFLINE.

- FR-4. Show type, weight and expiry on the OLED, marked as simulated. Use LEDs and the buzzer for success and for unknown-tag feedback.

- FR-5. Ignore a repeat scan of the same tag during a short cooldown.

### Cloud backend

- FR-6. Accept item registrations from the Fusion PC or a replay script over HTTPS.

- FR-7. Store products, pending parcels and batches.

- FR-8. On each scan, turn the pending parcel into a batch, record the arrival event and update stock.

- FR-9. Serve the daily forecast per product with p10, p50 and p90 values.

- FR-10. Compute an estimated sell date per batch and recompute it on every new arrival.

### Owner app

- FR-11. Show the forecast (with its likely range) vs. arrived for the day, per product.

- FR-12. Show stock by product and batch, with simulated expiry and estimated sell date.

- FR-13. Show a recent-arrivals feed that updates without a manual reload.

- FR-14. Flag batches at expiry risk.

## 7. Conceptual data model

| Entity | Key fields |
| --- | --- |
| Product | product_id, name, category, weight_kg (distinct per demo product), shelf_life_days |
| PendingParcel | parcel_id, tag_uid, product_id, weight_kg, registered_at, status (pending or received) |
| Batch | batch_id, product_id, tag_uid, quantity (units in the box), simulated expiry date, arrived_at |
| ArrivalEvent | event_id, device_id, tag_uid, batch_id, result (ok, unknown_tag, duplicate), timestamp |
| Forecast | date, product_id, p10, p50, p90 |
| SellEstimate | batch_id, estimated_sell_date, expiry_risk |

Expiry date = assumed manufacturing date + simulated shelf life, with small random variation between demo batches. It is never presented as real food-safety information.

## 8. Hardware (from the shared circuit)

| Component | Role | Status |
| --- | --- | --- |
| ESP32 dev board | Station controller, Wi-Fi to cloud | Real |
| MFRC522 RFID reader and tags | Identify the box | Real |
| OLED display (appears to be SSD1306) | Show scan result | Real |
| Push button | Possible stand-in for the arrival trigger (to confirm) | Real |
| Three LEDs (green, red, yellow) with resistors | Status feedback | Real |
| Buzzer | Audible feedback | Real |
| Servo | Visual indicator only, never blocks the flow | Optional |
| IR sensor | Arrival detection | Not visible in the circuit |

## 9. Non-functional requirements

- Demo reliability comes first: the golden path must work from a phone hotspot as well as venue Wi-Fi.

- Target under about 3 seconds from RFID read to the app updating [ASSUMPTION: my suggested target].

- Every simulated value (weight, expiry, forecast data) is visibly labeled as simulated.

- Separate keys: a read-only public key in the app, a limited device key on the ESP32, and a service key that never leaves laptop scripts and server functions.

- Warm up the backend shortly before the demo, since the first request after idle can be slow, and confirm the free Supabase project has not paused (it pauses after 7 days of inactivity).

## 10. Success criteria

- The golden-path demo runs end to end without manual database edits.

- At least two different boxes produce correct type, expiry, and sell-date results.

- The app visibly shows expected vs. arrived changing after a scan.

- The team can explain what is real and what is simulated in one sentence.

## 11. Risks

| Risk | Mitigation |
| --- | --- |
| Wi-Fi drops during the demo | Phone hotspot and an on-device table of known tags |
| First request after idle is slow (backend cold start) | Call a health check shortly before the demo and between runs |
| Free Supabase project pauses after 7 days of inactivity | Check it the day before and just before presenting |
| Fusion holds no item data or export, and outbound requests from it are unverified | Replay script first; a demo catalog file supplies item details |
| Two products share a weight and collide in the type table | Use distinct weights; the cloud is the source of truth for type |
| Forecaster needs rework (per-product series, forward horizon, saved output) and its runtime is unmeasured | Timebox it; fall back to precomputed forecast output |
| Secret keys exposed (.env files exist in the Fusion script folders) | Never commit .env files; keep the service key off the ESP32 and out of the app |
| Scope creep from AI and chat features | Keep them in Could until Must items work |

## 12. Assumptions and open items

- The forecasting repo has been reviewed: a LightGBM quantile model on one synthetic series that trains on every run and saves nothing. It needs per-product series, a forward horizon and a persistence step (see the TRD).

- Arrival trigger: the circuit has a push button and no IR sensor. Is the button a stand-in for IR, or will an IR sensor be wired in? Still open.

- The sell-date method (current stock against forecast daily demand, oldest batch first) is a suggestion pending the AI teammate's confirmation.

- Cloud choice is resolved: Supabase for everything, with no AWS.

- Fusion holds no item name, category, shelf life or tag UID and has no export. A demo catalog file supplies them. The path to the ESP32 goes through the cloud. Replay script first, live Fusion call second.

- Product type: the PRD assumes the cloud returns it and the ESP32 keeps a hard-coded weight-to-type table only as an offline fallback. The teammate's plan was to hard-code it on the ESP32, so this needs a team decision.

- Units: how many units does one box hold, and does the forecast's demand scale match? Without this the sell dates will not be meaningful.

- The ESP32's HTTPS certificate approach is still to be decided.

- The 10 kg rule now survives only as a visual in the Fusion animation and the optional servo indicator.